Overview

NonAdaptive demonstrates that a two-level control system can appear to adapt to changing characteristics of the environment without actually changing anything.

The system chosen for this demonstration is designed to control the position of a mass that is connected to the system via a spring and is damped as if moving through a viscous fluid.  The upper level senses the position of the mass, compares this to its current position, and uses the difference between them to set the reference level for the lower-level system, which controls the velocity of the mass.  It does so by generating a force on the mass whose direction (with or against the direction in which the mass is moving) and magnitude depend on the size of the error between the actual velocity of the mass and its current reference velocity.

During the demonstration, you can vary size of the mass, stiffness of the spring, and level of damping and observe how these variables affect the control system’s ability to bring the mass to rest after the reference position of the mass suddenly changes.  You can test the system’s response to this sudden step-change of target position while varying the output gains of the system’s two levels of control (position and velocity) and can set the size and direction of the step-change by manipulating the target position.  You can also have the program generate a set of three Bode (pronounced “BO-dee”) plots showing that the system behaves as if it were a simple proportional, integral, or double-integral control system depending on the characteristics of the imposed load (spring-only, viscosity-only, and mass-only, respectively).

The Time-plot Display

The graph in the Time Plot Panel (upper-right portion of the screen) displays the reference position of the mass (black), the actual position of the mass (red), its velocity (green), and the force being applied to it by the control system (blue), all as a function of time.  The plot is being redrawn several times per second, so that you can instantly see how these variables respond to changes in target position, the gains of the two-level control system, and the characteristics of the load (mass, spring, and damping).

Set the target position by means of the vertical slider control located in the Adjust Target panel to the immediate lower-right of the time-plot graph; this will change the position of the black line on the graph accordingly.  In the display, the target position remains at zero for a short time and then instantly steps to the position set for it.  The green and blue curves show how the position and velocity of the mass change as the control system brings the mass to its new target position.

The Control Panel (left side of screen) includes sub-panels containing slider control for adjusting the gains of the position and velocity control systems and for adjusting the load parameters.  Experiment with these values to see how they affect the output of the time plot.

The Bode Plot Display

The Bode Plots Panel located at bottom right displays or updates three Bode plots when you click on the button labeled (Update Bode Plots).  A Bode plot displays the amplitude (in decibels) and phase angle (in degrees) of a system’s output as a function of frequency (here scaled in radians/second) – how rapidly the target position varies.  To test the mass-spring-damper system, the target position is varied in a sine-wave pattern at various frequencies. Tthe mass will follow the target position most closely at the lowest frequencies, moving less as the frequency increases; this amplitude of movement is then recorded on the Bode plot.  In addition, the mass will tend to lag behind the target more as the frequency increases; this lag, expressed in degrees, is the second variable recorded on the Bode plot.

The three Bode plots show how the two-level control system responds to different test conditions involving specific combinations of load, springiness, and damping.  For this reason, the Mass, Spring, and Damping control positions are ignored during the generation of the Bode plots and have no effect on them.  However, the positions of the position and velocity gain controls do affect these results, and you can experiment with them to observe their effects on the plots.

The Proportional-Load Plot shows how this two-level control system functions when the load consists only of a strong spring (there's a minimum mass of 0.2 units or one percent of the maximum mass, representing the arm that moves the load; the damping is zero.).  The frequency response characteristics are very close to those of a single-integral controller.

The Single-Integral-Load Plot shows how the system functions when the load consists of damping with a very small mass and no spring.  The frequency response characteristics are very close to those of a proportional controller.

The Double-Integral-Load Plot shows how the system functions when the load consists of a large mass alone, with the spring removed and with no damping.  The response characteristics are very close to those of a first-derivative controller.

In all three cases, the overall effect is that of having a single integration somewhere in the control loop.

Although the frequency response characteristics change with load characteristics, it is important to remember that the parameters of the two-level control system are identical in each case.  It “adapts” to changing load conditions without adaptation. The only physical changes are in the load that is being moved; the system doing the controlling remains exactly the same.