PCT: The Negative Feedback Control Paradigm

Carey, Goldstein, Nevin, Powers,Taylor: 20100916

You see someone with a finger on a button beside a door. You ask yourself: “What is he doing?”, and the answer seems simple: “He’s ringing the doorbell”. But is that what the person thinks he is doing? Maybe he thinks he is trying to get someone to open the door. Maybe he thinks he is visiting Aunt Mary. Maybe he thinks he is trying to add to the expected vote total for his preferred candidate in an upcoming election. Maybe he thinks he is making some money by delivering pamphlets. Whatever he thinks he is doing, it probably is not “putting my finger on a button”.

This illustration may seem to belabour the obvious, that in everyday life, people have purposes, and that the purpose is what the person is “doing”. Equally obvious is that another person usually cannot determine a person’s purpose by observing the person’s actions. What is perhaps less obvious is that a purpose can be represented by a “what”, a “why”, and a “how”.

In the doorbell illustration, the first “what” that an observer might guess is “he is ringing the doorbell”. Its “why” is problematic, but its “how” is “by pressing the doorbell button”. However, even this “how” has its own “What-why-how” pattern. “What” is “seeing and feeling my finger pushing the button”, “why” is “to see myself pressing the button” and the “how” is “by moving my hand and arm to the appropriate place”.

If our observer looks at the possible “why” of pressing the doorbell, another pattern of “what-why-how” emerges, for which pressing the doorbell is the “how”. “What” might be “to get someone to open the door”, or put better “to see someone open the door”. And so it goes. Every “what someone is doing” is part of a “what-why-how” structure. In every case, “why” is because some state of the world is not as the person would like it to be, and “how” is a means of making the world a little or a lot more nearly as the person would wish.

All of this sounds self-evident, if not scientific. But it can be scientific. The “what-why-how” complex describes “control”, bringing some condition toward a desired state and maintaining it there. That is the engineering definition of control, and the thesis of this paper is that control is what living organisms do.

Since the early 1950s, a basic model of behavior (first published in 1960) has been under development. It was named Perceptual Control Theory (PCT) by members of an interdisciplinary study group that first came to its support during the 1980s. While the theory is quite simple, it departs sufficiently from consensus to have remained outside the mainstream of scientific thought. This paper is a summary of the PCT paradigm as it exists today. It is also a suggestion that PCT has now progressed far enough to claim its place under the sun.

Behavior as control

The basic thesis of PCT is not difficult to describe. Rather than being the final product of prior causes, the behavior of organisms is understood to be a collection of means to ends, the ends being control of the state of the local environment and ultimately of the organism itself. This requires that the basic organization of a living system be of the kind that is capable of controlling as opposed to  responding to inputs or generating patterned outputs (though both of those modes of action can be incorporated into the model). Furthermore, because actions are directed so as to create and maintain specific conditions of the organism and its environment, the logic of this kind of system is circular, cause and effect being subtly rearranged. What appear at first to be ordinary physical consequences of motor activities are seen as goals actively sought and defended against disturbances.

It is the capacity to control that, in PCT, marks a living system as different from other natural arrangements of matter and energy. The kind of control system described in PCT can have purposes of its own -- that is, it can spontaneously select as goals future states of the world around it and alter its own behavior to achieve and maintain such goal-states. It can automatically, without external guidance or instruction, adjust its actions to oppose the effects of random and otherwise unpredictable disturbances quickly and accurately enough to prevent their having any important effects. It can control hierarchically; that is, it can adjust one set of goals as a means of achieving higher-order goals. It can control many different variables in parallel at the same level of abstraction, and by that means control multiple variables of a higher order at the same time. It can learn and adapt: it can alter aspects of its own organization in ways that matter to it less in order to control variables that matter to it more.

In short, the PCT model is a drastic departure from what has been considered a scientific understanding of organisms for over 100 years.  In the past there has been an organized effort to present organisms as simply one more possible arrangement of matter and energy, subject to the same laws of physics and chemistry as any other arrangement. While PCT does not require any violation of the laws of physics and chemistry, it is specifically concerned with emergent laws that grow out of particular physical and chemical organizations. As long known, organisms have the ability to use the orderliness in the world around them as a means of increasing their own orderliness. They can impose order on their local worlds at the expense of order elsewhere.

Control does not imply totally arbitrary intervention in the processes of the environment, but it often seems to do so, in that the organism and its world both behave quite differently when the organism is in control. A car left to steer itself would soon run off the road or collide with another car, with only momentum, gravity, wind, and potholes affecting it. But add a driver to the car, and it -- and a huge number of other cars like it -- stays on the road, in its proper lane, for hundreds of miles and travels to a destination with high reliability. This is a highly improbable outcome when control is not taken into account. With control added to the picture, the same outcome becomes highly probable.

The PCT model departs from conventional understanding in another way. Rather than depicting behavior as a mechanistic outcome of exteral forces acting on an otherwise inert "preparation", the primary point of view is that  of the organism, not that of an external observer. The observer's subjective experience is as much a part of the model as any "objective" properties of control systems. This is the underlying reason for calling this theory perceptual control theory.

One of the more obvious facts is the fact that the external world is known to organisms only as it is represented inside them by electrical and chemical signals. That limitation applies not only to organisms in the wild and in the laboratory, but to the scientists who study them. Clearly, what we experience as the world around us must exist in the form of internal perceptions that only conjecturally and provisionally correspond to distinct entities outside us.

If all that we experience is in the form of internal perceptual signals, which is what neurology tells us, then all we can control is in the same form. In the course of controlling our own perceptions, we act on the world and alter it, so others can see us controlling aspects of their worlds, too. But we can never directly know what we are doing to that outside world as we control what we experience of it. To know that, we would have to experience the outer world without first filtering it through the senses. So far, that does not seem possible.

From PCT comes a view of human behavior and existence that is in better agreement with individual experience than the views that went before it. A clearly mechanistic model of behavior that can be implemented and studied as a computer simulation also explains how human beings can have goals, intentions, preferences. and desires, experiences sometimes thought to be figments of the imagination or simply errors of interpretation. The objective, engineering approach to behavior is brought together in PCT with subjective experience to create a richer picture of human existence than previously available.

The question naturally arises: if PCT has been building into a coherent model for 60 years or so, why doesn't everyone know about it? The answer may be that everyone does know about it, but that it has been misinterpreted and mischaracterized so much that its real potential has gone unrealized.

The premature end of PCT

The idea of negative feedback control was formalized by engineers in the 1930s, after having existed in many recorded forms since about 250 BC. By the 1940s it had given rise to the automation revolution of the World War Two years . The resemblance of negative feedback control systems to living systems brought forth cybernetics, and a revolution in the life sciences began to gather momentum in the late 1940s and early 1950s.

Within another ten years, the latter revolution was essentially dead;  negative feedback control was abandoned as a model of living systems by its original proponent, the prominent cyberneticist W. Ross Ashby.

In its place Ashby and others offered a different idea: the idea that organisms analyze the environment, determine what actions would be needed to produce desired results, and then issue the commands necessary to make the muscles generate those actions. This represented a return to the original idea of brain operation offered by Sherrington in 1906, in which the cerebral cortex formulated general commands that were then elaborated, level by level, into the detailed commands needed to produce organized behavior. Ashby showed persuasively that this more complex design would operate faster than the negative feedback control system, eliminating delays, and would be more accurate since it did not need to allow any errors to occur. It could even anticipate disturbances and generate actions to oppose them at the same instant they occurred.

From that time onward, negative feedback control has been regarded by many as old-fashioned, having been replaced by a more modern kind of control system that could achieve optimal control, the most accurate and swift control theoretically possible, at the least cost. Since evolution would naturally have shaped organisms to operate in the best possible way, it was assumed that this model should also be used to explain the behavior of organisms, at least those with sufficient complexity to carry out the required analyses and matrix inversions.

There was only one drawback to this advance in control theory when used as a model of organisms: real organisms seldom behave that way. It is, in fact, easy to design artificial control systems that control much better than organisms do. To make a model that behaves as much as possible the way a real person does in, for example, a tracking task, it is necessary to resurrect the negative feedback control model. Ashby had the right idea the first time, and threw it away.

In the following demonstration of a negative feedback control model (Powers 2008), a person uses a mouse to make a cursor track a moving target for one minute. Data are sampled 60 times per second. The data for the experimental run are shown in the upper plot of Fig. 1. The red trace shows the target movements; the green trace shows the mouse and cursor movements. The black trace shows the difference between target and cursor -- the tracking error.

Even to the naked eye, it's clear that there is a time delay between target movements and cursor movements. The delay is not removed by anticipatory mouse movements. In the upper part of Fig. 1 the results of fitting a negative feedback control model to the data are summarized; the best-fit delay in the model's response is 7/60ths of a second, which is 7 frames of the computer display running at 60 Hz, or 116 milliseconds. That is how far behind the target movements the participant is moving the cursor.

Fig. 1. Analysis of human tracking run and fit of negative feedback control model to the data.

In the lower plot, the model's simulated mouse movements (blue) are compared with the real mouse movements (green). They are very nearly identical, with the same delay relative to the target movements. The mean difference between model and real behavior is 3.6% of the range of target movement. In this run, the target movements are rapid enough that the tracking error is 9% of the target range; the model fits the real data well within the tracking error, showing that the model is making similar mistakes. The model accounts for at least 84% of the deviations the human makes from the ideal track, whereas a perfect control system would account for none of the human’s errors.

Aristotle once declared that men have more teeth than women do. This proposition was based on philosophical principles and arguments, experimentation being considered an inferior approach in his time. However, real men do not have more teeth than women, whatever logical reasoning may say. When we look at actual human data, we find that the analyze-calculate-execute idea is also mainly a product of logical reasoning.

Learning from computer demonstrations